# ATNT

Network Vulnerability Assessment Report

Table of Contents

- Executive Summary	3

- Scan Results	3

- Our Findings	4

- Risk Assessment	4

Critical Severity Vulnerability	4

High Severity Vulnerability	5

Medium Severity Vulnerability	5

Low Severity Vulnerability	6

- Recommendations	7

Remediation	7

# Executive Summary

This document reports on the results of an automatic security scan. All dates are dis-

played using the timezone Coordinated Universal Time, which is abbreviated as UTC. The

task was an Immediate scan of IP 10.0.60.0/24. The scan started at Wednesday, May 7 01:34:48

2025 UTC and ended on Wednesday, May 7 03:04:32 2025 UTC. The report summarizes the

results found after scanning the network. Then, for each host, the report describes every issue found. Please consider the advice given in each description, in order to resolve any issues.

# Scan Results

#### *The raw scan results will be provided upon delivery*. (*INSERT IMAGE OF SCAN*)

# Our Findings

The results from the credentialed patch audit are listed below. It is important to note that not all identified hosts were able to be scanned during this assessment - of the 10.0.60.254 hosts identified as belonging to the “BlackBoxOps” domain, only three hosts were successfully scanned. In addition, some of the hosts that were successfully scanned were not included in the host list provided.

# Risk Assessment

This report identifies security risks that could have significant impact on mission-critical applications used for day-to-day business operations.

| Critical | High | Medium | Low |
| --- | --- | --- | --- |
| 1 | 0 | 5 | 1 |

## Critical Severity Vulnerability

One unique critical severity vulnerability. Critical vulnerabilities require immediate attention. They are relatively easy for attackers to exploit and may provide them with full control of the affected systems.

A table of the top critical severity vulnerabilities is provided below:

| ID | DESCRIPTION | CVE | Severity | MITIGATION | CVE/Info Links |
| --- | --- | --- | --- | --- | --- |
| 1.3.6.1.4.1.25623.1.0.105937 | The Operating System (OS) on the remote host has reached the end of life (EOL) and should not be used anymore. | N/A | Critical 10 | Upgrade the OS on the host to a version which is still supported and receiving security updates by the vendor. Or choose a new OS to use instead | https://www.securityspace.com/smysecure/catid.html?id=1.3.6.1.4.1.25623.1.0.105937 |

## High Severity Vulnerability

(0 ) high severity vulnerabilities were found. High severity vulnerabilities are often harder to exploit and may not provide the same access to affected systems.

## Medium Severity Vulnerability

(5) were unique medium severity vulnerabilities. These vulnerabilities often provide information to attackers that may assist them in mounting subsequent attacks on your network. These should also be fixed in a timely manner but are not as urgent when compared to other vulnerabilities.

A table of the top high severity vulnerabilities is provided below:

| ID | DESCRIPTION | CVE | Severity | MITIGATION | CVE/Info Links |
| --- | --- | --- | --- | --- | --- |
| 1.3.6.1.4.1.25623.1.0.105565 | The remote SSH server is configured to allow / support weak host key algorithm(s) | N/A | Medium (5.3) | Disable the reported weak host key algorithm(s) | https://www.rfc-editor.org/rfc/rfc8332 url: https://www.rfc-editor.org/rfc/rfc8709 https://www.rfc-editor.org/rfc/rfc4253#section-6.6 |
| 1.3.6.1.4.1.25623.1.0.150713 | The remote SSH server is configured to allow / support weak key exchange (KEX) algorithm(s). | ## CVE-2016-0861 CVE-2016-0862 | Medium 5.3 | Disable the reported weak KEX algorithm(s) - 1024-bit MODP group / prime KEX algorithms: Alternatively use elliptic-curve Die-Hellmann in general, e.g. Curve 25519. | https://weakdh.org/sysadmin.html https://www.rfc-editor.org/rfc/rfc9142 https://www.rfc-editor.org/rfc/rfc9142#name-summary-guidance-for-implem https://www.rfc-editor.org/rfc/rfc6194 https://www.rfc-editor.org/rfc/rfc4253#section-6.5 https://nvd.nist.gov/vuln/detail/CVE-2016-0861 https://nvd.nist.gov/vuln/detail/CVE-2016-0862 |
| 1.3.6.1.4.1.25623.1.0.105565 | The remote SSH server is configured to allow / support weak encryption algorithm(s). | N/A | Medium 4.3 | Disable the reported weak encryption algorithm(s) | https://www.rfc-editor.org/rfc/rfc8758 https://www.kb.cert.org/vuls/id/958563 https://www.rfc-editor.org/rfc/rfc4253#section-6.3 |
| 1.3.6.1.4.1.25623.1.0.10736 | Distributed Computing Environment / Remote Procedure Calls (DCE/RPC) or MSRPC ser- vices running on the remote host can be enumerated by connecting on port 135 and doing the appropriate queries. | N/A | Medium 2.0 | Filter incoming traffic to these ports. | https://secure1.securityspace.com/smysecure/catid.html?id=1.3.6.1.4.1.25623.1.0.10736 |
| 1.3.6.1.4.1.25623.1.0.108522 | The remote host is running a Telnet service that allows cleartext logins over unencrypted connections | CVE-2014-0224 | Medium 4.8 | Replace Telnet with a protocol like SSH which supports encrypted connections. | https://secure1.securityspace.com/smysecure/catid.html?id=1.3.6.1.4.1.25623.1.0.10736 https://nvd.nist.gov/vuln/detail/CVE-2014-0224 |

## Low Severity Vulnerability

1 low severity vulnerabilities were found during this scan.

| ID | DESCRIPTION | CVE | Severity | MITIGATION | CVE/Info Links |
| --- | --- | --- | --- | --- | --- |
| 1.3.6.1.4.1.25623.1.0.105565 | The remote SSH server is configured to allow / support weak MAC algorithm(s). | N/A | Low 2.6 | Disable the reported weak MAC algorithm(s) | https://www.rfc-editor.org/rfc/rfc6668 https://www.rfc-editor.org/rfc/rfc4253#section-6.4 |

# Recommendations

Recommendations in this report are based on the available findings from the credentialed patch audit. Vulnerability scanning is only one tool to assess the security posture of a network. The results should not be interpreted as definitive measurement of the security posture of the BlackBoxOps network. Other elements used to assess the current security posture would include policy review, a review of internal security controls and procedures, or internal penetration testing.

### Remediation

Taking the following actions across all hosts will resolve 96% of the vulnerabilities on the network:

| ACTION TO TAKE | VULNS | HOSTS |
| --- | --- | --- |
| Upgrade the OS on the remote host to a version which is still supported and receiving security updates by the vendor. | 1 | 1 |
| Disable the reported weak host key/encryption algorithm(s). | 2 | 1 |
| Disable the weak KEX algorithm(s) | 1 | 1 |
| Disable the weak MAC algorithm(s) | 1 | 1 |
| Filter incoming traffic to ports 135. | 1 | 1 |
| Replace Telnet with a protocol like SSH which supports encrypted connections | 1 | 1 |

I Summary

=========

This document reports on the results of an automatic security scan.

The report first summarises the results found.

Then, for each host, the report describes every issue found.

Please consider the advice given in each description, in order to rectify

the issue.

All dates are displayed using the timezone "Coordinated Universal Time",

which is abbreviated "UTC".

Vendor security updates are not trusted.

Overrides are off.  Even when a result has an override, this report uses

the actual threat of the result.

Notes are included in the report.Information on overrides is included in the report.

This report might not show details of all issues that were found.

Issues with the threat level "Log" are not shown.

Issues with the threat level "Debug" are not shown.

Issues with the threat level "False Positive" are not shown.

Only results with a minimum QoD of 70 are shown.

This report contains all 7 results selected by the

filtering described above.  Before filtering there were 167 results.

Scan started: Wed May 7 01:34:48 2025 UTC

Scan ended:   Wed May 7 03:04:32 2025 UTC

Task:     	Immediate scan of IP 10.0.60.0/24

Host Summary

************

Host        	High  Medium  Low  Log  False Positive

10.0.60.85     	1   	3	1	0           	0

10.0.60.107    	0   	1	0	0           	0

10.0.60.254    	0   	1	0	0           	0

Total: 3       	1   	5	1	0           	0

II Results per Host

===================

Host 10.0.60.85

***************

Scanning of this host started at: Wed May 7 01:37:27 2025 UTC

Number of results: 5

Port Summary for Host 10.0.60.85

--------------------------------

Service (Port)      	Threat Level

general/tcp         	High (CVSS: 10.0)

22/tcp              	Medium (CVSS: 5.3)

Security Issues for Host 10.0.60.85

-----------------------------------

Issue

-----

NVT:	Operating System (OS) End of Life (EOL) Detection

OID:	1.3.6.1.4.1.25623.1.0.103674

Threat: High (CVSS: 10.0)

Port:   general/tcp

Product detection result: cpe:/o:debian:debian_linux:7

Detected by: OS Detection Consolidation and Reporting (OID: 1.3.6.1.4.1.25623.1.0.105937)

Summary:

The Operating System (OS) on the remote host has reached the end

of life (EOL) and should not be used anymore.

Vulnerability Detection Result:

The "Debian GNU/Linux" Operating System on the remote host has reached the end o!

f life.

CPE:           	cpe:/o:debian:debian_linux:7

Installed version,

build or SP:   	7

EOL date:      	2018-05-31

EOL info:      	https://en.wikipedia.org/wiki/List_of_Debian_releases#Release!

_table

Impact:

An EOL version of an OS is not receiving any security updates

from the vendor. Unfixed security vulnerabilities might be leveraged by an att!

acker to compromise

the security of this host.

Solution:

Solution type: Mitigation

Upgrade the OS on the remote host to a version which is still

supported and receiving security updates by the vendor.

Vulnerability Detection Method:

Checks if an EOL version of an OS is present on the target

host.

Details:

Operating System (OS) End of Life (EOL) Detection

(OID: 1.3.6.1.4.1.25623.1.0.103674)

Version used: 2025-04-15T05:54:49Z

Product Detection Result:

Product:cpe:/o:debian:debian_linux:7

Method:OS Detection Consolidation and Reporting

(OID: 1.3.6.1.4.1.25623.1.0.105937)

Issue

-----

NVT:	Weak Host Key Algorithm(s) (SSH)

OID:	1.3.6.1.4.1.25623.1.0.117687

Threat: Medium (CVSS: 5.3)

Port:   22/tcp

Product detection result: cpe:/a:ietf:secure_shell_protocol

Detected by: SSH Protocol Algorithms Supported (OID: 1.3.6.1.4.1.25623.1.0.105565)

Summary:

The remote SSH server is configured to allow / support weak host

key algorithm(s).

Vulnerability Detection Result:

The remote SSH server supports the following weak host key algorithm(s):

host key algorithm | Description

--------------------------------------------------------------------------------!

---------

ssh-dss        	| Digital Signature Algorithm (DSA) / Digital Signature Stand!

ard (DSS)

Solution:

Solution type: Mitigation

Disable the reported weak host key algorithm(s).

Vulnerability Detection Method:

Checks the supported host key algorithms of the remote SSH

server.

Currently weak host key algorithms are defined as the following:

- ssh-dss: Digital Signature Algorithm (DSA) / Digital Signature Standard (DSS!

)

Details:

Weak Host Key Algorithm(s) (SSH)

(OID: 1.3.6.1.4.1.25623.1.0.117687)

Version used: 2024-06-14T05:05:48Z

Product Detection Result:

Product:cpe:/a:ietf:secure_shell_protocol

Method:SSH Protocol Algorithms Supported

(OID: 1.3.6.1.4.1.25623.1.0.105565)

References:

url: https://www.rfc-editor.org/rfc/rfc8332

url: https://www.rfc-editor.org/rfc/rfc8709

url: https://www.rfc-editor.org/rfc/rfc4253#section-6.6

Issue

-----

NVT:	Weak Key Exchange (KEX) Algorithm(s) Supported (SSH)

OID:	1.3.6.1.4.1.25623.1.0.150713

Threat: Medium (CVSS: 5.3)

Port:   22/tcp

Product detection result: cpe:/a:ietf:secure_shell_protocol

Detected by: SSH Protocol Algorithms Supported (OID: 1.3.6.1.4.1.25623.1.0.105565)

Summary:

The remote SSH server is configured to allow / support weak key

exchange (KEX) algorithm(s).

Vulnerability Detection Result:

The remote SSH server supports the following weak KEX algorithm(s):

KEX algorithm                  	| Reason

--------------------------------------------------------------------------------!

-----------

diffie-hellman-group-exchange-sha1 | Using SHA-1

diffie-hellman-group1-sha1     	| Using Oakley Group 2 (a 1024-bit MODP group!

) and SHA-1

Impact:

An attacker can quickly break individual connections.

Solution:

Solution type: Mitigation

Disable the reported weak KEX algorithm(s)

- 1024-bit MODP group / prime KEX algorithms:

Alternatively use elliptic-curve Diffie-Hellmann in general, e.g. Curve 25519.

Vulnerability Insight:

- 1024-bit MODP group / prime KEX algorithms:

Millions of HTTPS, SSH, and VPN servers all use the same prime numbers for Dif!

fie-Hellman key

exchange. Practitioners believed this was safe as long as new key exchange mes!

sages were generated

for every connection. However, the first step in the number field sieve-the mo!

st efficient

algorithm for breaking a Diffie-Hellman connection-is dependent only on this p!

rime.

A nation-state can break a 1024-bit prime.

Vulnerability Detection Method:

Checks the supported KEX algorithms of the remote SSH server.

Currently weak KEX algorithms are defined as the following:

- non-elliptic-curve Diffie-Hellmann (DH) KEX algorithms with 1024-bit MODP gr!

oup / prime

- ephemerally generated key exchange groups uses SHA-1

- using RSA 1024-bit modulus key

Details:

Weak Key Exchange (KEX) Algorithm(s) Supported (SSH)

(OID: 1.3.6.1.4.1.25623.1.0.150713)

Version used: 2024-06-14T05:05:48Z

Product Detection Result:

Product:cpe:/a:ietf:secure_shell_protocol

Method:SSH Protocol Algorithms Supported

(OID: 1.3.6.1.4.1.25623.1.0.105565)

References:

url: https://weakdh.org/sysadmin.html

url: https://www.rfc-editor.org/rfc/rfc9142

url: https://www.rfc-editor.org/rfc/rfc9142#name-summary-guidance-for-implem

url: https://www.rfc-editor.org/rfc/rfc6194

url: https://www.rfc-editor.org/rfc/rfc4253#section-6.5

Issue

-----

NVT:	Weak Encryption Algorithm(s) Supported (SSH)

OID:	1.3.6.1.4.1.25623.1.0.105611

Threat: Medium (CVSS: 4.3)

Port:   22/tcp

Product detection result: cpe:/a:ietf:secure_shell_protocol

Detected by: SSH Protocol Algorithms Supported (OID: 1.3.6.1.4.1.25623.1.0.105565)

Summary:

The remote SSH server is configured to allow / support weak

encryption algorithm(s).

Vulnerability Detection Result:

The remote SSH server supports the following weak client-to-server encryption al!

gorithm(s):

3des-cbc

aes128-cbc

aes192-cbc

aes256-cbc

arcfour

arcfour128

arcfour256

blowfish-cbc

cast128-cbc

rijndael-cbc@lysator.liu.se

The remote SSH server supports the following weak server-to-client encryption al!

gorithm(s):

3des-cbc

aes128-cbc

aes192-cbc

aes256-cbc

arcfour

arcfour128

arcfour256

blowfish-cbc

cast128-cbc

rijndael-cbc@lysator.liu.se

Solution:

Solution type: Mitigation

Disable the reported weak encryption algorithm(s).

Vulnerability Insight:

- The 'arcfour' cipher is the Arcfour stream cipher with 128-bit

keys. The Arcfour cipher is believed to be compatible with the RC4 cipher [SCH!

NEIER]. Arcfour

(and RC4) has problems with weak keys, and should not be used anymore.

- The 'none' algorithm specifies that no encryption is to be done. Note that t!

his method provides

no confidentiality protection, and it is NOT RECOMMENDED to use it.

- A vulnerability exists in SSH messages that employ CBC mode that may allow a!

n attacker to

recover plaintext from a block of ciphertext.

Vulnerability Detection Method:

Checks the supported encryption algorithms (client-to-server

and server-to-client) of the remote SSH server.

Currently weak encryption algorithms are defined as the following:

- Arcfour (RC4) cipher based algorithms

- 'none' algorithm

- CBC mode cipher based algorithms

Details:

Weak Encryption Algorithm(s) Supported (SSH)

(OID: 1.3.6.1.4.1.25623.1.0.105611)

Version used: 2024-06-14T05:05:48Z

Product Detection Result:

Product:cpe:/a:ietf:secure_shell_protocol

Method:SSH Protocol Algorithms Supported

(OID: 1.3.6.1.4.1.25623.1.0.105565)

References:

url: https://www.rfc-editor.org/rfc/rfc8758

url: https://www.kb.cert.org/vuls/id/958563

url: https://www.rfc-editor.org/rfc/rfc4253#section-6.3

Issue

-----

NVT:	Weak MAC Algorithm(s) Supported (SSH)

OID:	1.3.6.1.4.1.25623.1.0.105610

Threat: Low (CVSS: 2.6)

Port:   22/tcp

Product detection result: cpe:/a:ietf:secure_shell_protocol

Detected by: SSH Protocol Algorithms Supported (OID: 1.3.6.1.4.1.25623.1.0.105565)

Summary:

The remote SSH server is configured to allow / support weak MAC

algorithm(s).

Vulnerability Detection Result:

The remote SSH server supports the following weak client-to-server MAC algorithm!

(s):

hmac-md5

hmac-md5-96

hmac-sha1-96

hmac-sha2-256-96

hmac-sha2-512-96

umac-64@openssh.com

The remote SSH server supports the following weak server-to-client MAC algorithm!

(s):

hmac-md5

hmac-md5-96

hmac-sha1-96

hmac-sha2-256-96

hmac-sha2-512-96

umac-64@openssh.com

Solution:

Solution type: Mitigation

Disable the reported weak MAC algorithm(s).

Vulnerability Detection Method:

Checks the supported MAC algorithms (client-to-server and

server-to-client) of the remote SSH server.

Currently weak MAC algorithms are defined as the following:

- MD5 based algorithms

- 96-bit based algorithms

- 64-bit based algorithms

- 'none' algorithm

Details:

Weak MAC Algorithm(s) Supported (SSH)

(OID: 1.3.6.1.4.1.25623.1.0.105610)

Version used: 2024-06-14T05:05:48Z

Product Detection Result:

Product:cpe:/a:ietf:secure_shell_protocol

Method:SSH Protocol Algorithms Supported

(OID: 1.3.6.1.4.1.25623.1.0.105565)

References:

url: https://www.rfc-editor.org/rfc/rfc6668

url: https://www.rfc-editor.org/rfc/rfc4253#section-6.4

Host 10.0.60.107

****************

Scanning of this host started at: Wed May 7 01:37:27 2025 UTC

Number of results: 1

Port Summary for Host 10.0.60.107

---------------------------------

Service (Port)      	Threat Level

135/tcp             	Medium (CVSS: 5.0)

Security Issues for Host 10.0.60.107

------------------------------------

Issue

-----

NVT:	DCE/RPC and MSRPC Services Enumeration Reporting

OID:	1.3.6.1.4.1.25623.1.0.10736

Threat: Medium (CVSS: 5.0)

Port:   135/tcp

Summary:

Distributed Computing Environment / Remote Procedure Calls (DCE/RPC) or MSRPC se!

rvices running

on the remote host can be enumerated by connecting on port 135 and doing the a!

ppropriate queries.

Vulnerability Detection Result:

Here is the list of DCE/RPC or MSRPC services running on this host via the TCP p!

rotocol:

Port: 49664/tcp

UUID: 12345778-1234-abcd-ef00-0123456789ac, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49664]

Named pipe : lsass

Win32 service or process : lsass.exe

Description : SAM access

UUID: 51a227ae-825b-41f2-b4a9-1ac9557a1018, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49664]

Annotation: Ngc Pop Key Service

UUID: 8fb74744-b2ff-4c00-be0d-9ef9a191fe1b, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49664]

Annotation: Ngc Pop Key Service

UUID: b25a52bf-e5dd-4f4a-aea6-8ca7272a0e86, version 2

Endpoint: ncacn_ip_tcp:10.0.60.107[49664]

Annotation: KeyIso

Port: 49665/tcp

UUID: d95afe70-a6d5-4259-822e-2c84da1ddb0d, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49665]

Port: 49666/tcp

UUID: f6beaff7-1e19-4fbb-9f8f-b89e2018337c, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49666]

Annotation: Event log TCPIP

Port: 49667/tcp

UUID: 3a9ef155-691d-4449-8d05-09ad57031823, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49667]

UUID: 86d35949-83c9-4044-b424-db363231fd0c, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49667]

Port: 49671/tcp

UUID: 0b6edbfa-4a24-4fc6-8a23-942b1eca65d1, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49671]

UUID: 12345678-1234-abcd-ef00-0123456789ab, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49671]

Named pipe : spoolss

Win32 service or process : spoolsv.exe

Description : Spooler service

UUID: 4a452661-8290-4b36-8fbe-7f4093a94978, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49671]

UUID: 76f03f96-cdfd-44fc-a22c-64950a001209, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49671]

UUID: ae33069b-a2a8-46ee-a235-ddfd339be281, version 1

Endpoint: ncacn_ip_tcp:10.0.60.107[49671]

Port: 49673/tcp

UUID: 367abb81-9844-35f1-ad32-98f038001003, version 2

Endpoint: ncacn_ip_tcp:10.0.60.107[49673]

Note: DCE/RPC or MSRPC services running on this host locally were identified. Re!

porting this list is not enabled by default due to the possible large size of th!

is list. See the script preferences to enable this reporting.

Impact:

An attacker may use this fact to gain more knowledge

about the remote host.

Solution:

Solution type: Mitigation

Filter incoming traffic to this ports.

Vulnerability Detection Method:

Details:

DCE/RPC and MSRPC Services Enumeration Reporting

(OID: 1.3.6.1.4.1.25623.1.0.10736)

Version used: 2022-06-03T10:17:07Z

Host 10.0.60.254

****************

Scanning of this host started at: Wed May 7 01:37:27 2025 UTC

Number of results: 1

Port Summary for Host 10.0.60.254

---------------------------------

Service (Port)      	Threat Level

23/tcp              	Medium (CVSS: 4.8)

Security Issues for Host 10.0.60.254

------------------------------------

Issue

-----

NVT:	Telnet Unencrypted Cleartext Login

OID:	1.3.6.1.4.1.25623.1.0.108522

Threat: Medium (CVSS: 4.8)

Port:   23/tcp

Summary:

The remote host is running a Telnet service that allows cleartext logins over

unencrypted connections.

Vulnerability Detection Result:

Vulnerability was detected according to the Vulnerability Detection Method.

Impact:

An attacker can uncover login names and passwords by sniffing traffic to the

Telnet service.

Solution:

Solution type: Mitigation

Replace Telnet with a protocol like SSH which supports encrypted connections.

Vulnerability Detection Method:

Details:

Telnet Unencrypted Cleartext Login

(OID: 1.3.6.1.4.1.25623.1.0.108522)

Version used: 2023-10-13T05:06:09Z